# Design & Implementation of a Custom FPGA-Based Graphics Processing Unit (GPU)

## "Find the Thief" — 2D Mystery Game with AI-Driven NPC Dialogue

Team Members: Tutika Sandeep, Ayush, Namburi Venkata Yeshwanth, Ghaffar Salman, Faisal Mohammed

Supervisor: Dr. Bhanu Prakash

Institution: School of Technology, Woxsen University

Hardware: ESP32-S3-DevKitC-1 + Sipeed Tang Nano 20K

Display: HDMI 640x480 @ 60Hz

Input: Cosmic Byte Blitz Gamepad (USB)

AI Engine: nanoGPT ~4.8M parameters (INT8 on ESP32-S3)

# Part 1: Project Specification

# Custom FPGA GPU — FINAL Project Specification (v3)

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## Project Identity

| Field | Details |
| --- | --- |
| **Title** | Design & Implementation of a Custom FPGA-Based Graphics Processing Unit (GPU) |
| **Team** | Tutika Sandeep, Ayush, Namburi Venkata Yeshwanth, Ghaffar Salman, Faisal Mohammed |
| **Supervisor** | Dr. Bhanu Prakash |
| **Institution** | School of Technology, Woxsen University |
| **Game** | "Find the Thief" — 2D mystery village game with AI-driven NPC dialogue |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## Final Hardware BOM (Bill of Materials)

| # | Component | Model | Role |
| --- | --- | --- | --- |
| 1 | Host CPU | **ESP32-S3-DevKitC-1** (dual USB-C) | Game logic, AI inference, gamepad input |
| 2 | FPGA GPU | **Sipeed Tang Nano 20K** (GW2AR-18) | Rendering engine, HDMI output |
| 3 | Gamepad | **Cosmic Byte Blitz** | Player input |
| 4 | Adapter | **USB-A to USB-C adapter** | Connects gamepad to ESP32-S3 |
| 5 | Display | **Any HDMI monitor** | 640×480 @ 60Hz output |
| 6 | SD Card | **microSD card** (any, 4GB+) | Stores AI model weights + game assets |
| 7 | Cables | Jumper wires (male-male) | SPI connection between ESP32-S3 ↔ Tang Nano 20K |
| 8 | Power | USB-C cables (×2) | One for each board |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## Engineering Decisions (Made by Me)

### Decision 1: Color Depth → 8-bit Indexed (256-Color Palette)

Why this is the right choice for your project:

| Factor | 8-bit Indexed | 16-bit Direct | Verdict |
| --- | --- | --- | --- |
| VRAM per frame | 300 KB | 600 KB | 8-bit wins — half the memory |
| Double buffer | 600 KB | 1.2 MB | 8-bit uses 7.5% of SDRAM vs 15% |
| FPGA complexity | Simple palette LUT in BRAM | Direct RGB mapping | 8-bit simpler to implement |
| Art style match | Perfect for 2D pixel-art village | Overkill for this aesthetic | 8-bit matches the game |
| Palette tricks | Can do fade effects by changing palette | No palette tricks possible | 8-bit more versatile |

How it works: Each pixel stores a number 0–255. That number is an index into a 256-entry Color Lookup Table (CLUT) stored in BRAM. Each entry holds a full 18-bit or 24-bit RGB color. This gives you 256 simultaneous colors on screen from a total palette of 16 million.

### Decision 2: AI Model Execution → Runs on ESP32-S3 (NOT the FPGA)

Why the ESP32-S3, not the FPGA:

| Factor | ESP32-S3 | FPGA (Tang Nano 20K) |
| --- | --- | --- |
| Architecture | Sequential CPU — perfect for transformer's layer-by-layer inference | Parallel logic gates — designed for pixel-pushing |
| Memory access | Can stream weights from PSRAM/SD card sequentially | SDRAM is shared with framebuffer — contention risk |
| Proven feasibility | People have run 15M+ param models on ESP32-S3 | No known transformer inference on 20K LUT FPGA |
| Development effort | Use ESP-DL or TFLite Micro frameworks | Would require custom Verilog matrix multiply engine |
| Latency tolerance | NPC dialogue has 1–2 seconds of acceptable wait time | GPU needs sub-microsecond response for 60 FPS |

The AI pipeline:

Player asks NPC a question
        │
        ▼
┌─────────────────────────────────────────────────┐
│  ESP32-S3 (Core 0 — AI Inference)               │
│                                                  │
│  1. Build prompt: NPC personality + trust level   │
│     + memory + player's question                 │
│  2. Load model weights from SD card → PSRAM      │
│  3. Run nanoGPT inference (INT8 quantized)       │
│  4. Generate response tokens (~1-2 sec)          │
│  5. Parse response into game action + dialogue   │
└──────────────────┬──────────────────────────────┘
                   │
                   ▼
┌─────────────────────────────────────────────────┐
│  ESP32-S3 (Core 1 — Game Loop, always running)   │
│                                                  │
│  1. Receive NPC response text                    │
│  2. Update trust/reputation variables            │
│  3. Send "Draw Dialogue Box" command to FPGA     │
│  4. Send character sprite data to FPGA           │
└──────────────────┬──────────────────────────────┘
                   │ SPI + DMA
                   ▼
          ┌─────────────────┐
          │   FPGA (GPU)    │
          │   Renders the   │
          │   dialogue box  │
          │   and sprites   │
          └─────────────────┘

nanoGPT specs for your project:

| Parameter | Value |
| --- | --- |
| Architecture | Transformer (decoder-only, GPT-2 style) |
| Parameters | ~5 million |
| Precision | INT8 quantized |
| Model size on disk | ~5 MB |
| Storage | microSD card → loaded into PSRAM at boot |
| Runtime memory | ~5–6 MB in PSRAM during inference |
| Training data | Custom dialogue corpus (you'll curate this) |
| Inference speed | ~1–3 tokens/second (estimated) |
| Framework | ESP-DL or custom C inference engine |

### Decision 3: Gamepad Connection → USB Wired via OTG

Cosmic Byte Blitz ──[USB-A cable]──► USB-A to USB-C adapter ──► ESP32-S3 Native USB Port
                                                                  (labeled "USB", not "UART")

| Detail | Value |
| --- | --- |
| Connection type | USB wired (most reliable, lowest latency) |
| ESP32-S3 port | **Native USB-OTG** port (the one labeled "USB") |
| Library | `EspUsbHost` by tanakamasayuki |
| Protocol | USB HID (Human Interface Device) |
| Polling rate | Up to 1000 Hz |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## Final System Architecture

┌───────────────────────┐
                    │   Cosmic Byte Blitz   │
                    │   (USB HID Gamepad)   │
                    └──────────┬────────────┘
                               │ USB-A → USB-C adapter
                               │ (Native USB-OTG port)
                               ▼
┌──────────────────────────────────────────────────────┐
│              ESP32-S3-DevKitC-1                      │
│              (Host CPU + AI Brain)                   │
│                                                      │
│  ┌─────────────────┐    ┌──────────────────────┐     │
│  │ Core 1 (240MHz) │    │ Core 0 (240MHz)      │     │
│  │                 │    │                      │     │
│  │ • Game loop     │    │ • nanoGPT inference  │     │
│  │ • Input polling │    │ • INT8, ~5M params   │     │
│  │ • State machine │    │ • NPC personality    │     │
│  │ • Draw commands │    │ • Trust/reputation   │     │
│  └────────┬────────┘    └──────────┬───────────┘     │
│           │                        │                 │
│           │    ┌───────────┐       │                 │
│           │    │ microSD   │◄──────┘                 │
│           │    │ Card      │ Model weights           │
│           │    │ (AI + Art)│ + sprite sheets          │
│           │    └───────────┘                         │
│           │                                          │
│  ┌────────▼──────────┐                               │
│  │ SPI Master + DMA  │                               │
│  │ Binary draw cmds  │                               │
│  └────────┬──────────┘                               │
└───────────┼──────────────────────────────────────────┘
            │ SPI Bus (MOSI, MISO, SCLK, CS)
            │ + 4 jumper wires
            ▼
┌──────────────────────────────────────────────────────┐
│          Sipeed Tang Nano 20K                        │
│          (FPGA GPU — GW2AR-18)                       │
│                                                      │
│  ┌──────────────┐  ┌──────────────┐  ┌────────────┐ │
│  │ SPI Slave +  │  │ Instruction  │  │ Hardware   │ │
│  │ Cmd Buffer   │─►│ Decoder FSM  │─►│ Blitter    │ │
│  └──────────────┘  └──────┬───────┘  └─────┬──────┘ │
│                           │                │        │
│                    ┌──────▼───────┐  ┌─────▼──────┐ │
│                    │ Rasterizer   │  │ SDRAM      │ │
│                    │ (Bresenham)  │  │ Controller │ │
│                    └──────┬───────┘  └──┬───┬─────┘ │
│                           │             │   │       │
│                    ┌──────▼─────────────▼┐  │       │
│                    │  Frame Buffer       │  │       │
│                    │  (in 64Mb SIP SDRAM)│  │       │
│                    └──────────┬──────────┘  │       │
│                               │             │       │
│                    ┌──────────▼──────────┐  │       │
│                    │ Palette LUT (BRAM)  │  │       │
│                    │ 256 × 24-bit RGB    │  │       │
│                    └──────────┬──────────┘  │       │
│                               │             │       │
│                    ┌──────────▼──────────┐  │       │
│                    │ VGA Timing Gen      │  │       │
│                    │ 640×480 @ 60Hz      │  │       │
│                    └──────────┬──────────┘  │       │
│                               │             │       │
│                    ┌──────────▼──────────┐  │       │
│                    │ TMDS Encoder        │  │       │
│                    │ (8b→10b per channel)│  │       │
│                    └──────────┬──────────┘  │       │
│                               │             │       │
└───────────────────────────────┼─────────────┘       │
                                │ HDMI (TMDS)          
                                ▼                      
                    ┌───────────────────────┐           
                    │   HDMI Monitor        │           
                    │   640×480 @ 60Hz      │           
                    └───────────────────────┘

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## Memory Map

### ESP32-S3 Memory

| Memory | Size | Contents |
| --- | --- | --- |
| Internal SRAM | 512 KB | Game variables, NPC states, trust arrays, active buffers |
| PSRAM | 8 MB | nanoGPT weights (loaded from SD), inference workspace |
| microSD Card | 4+ GB | AI model file (~5MB), sprite sheet PNGs, dialogue corpus, map data |

### FPGA (Tang Nano 20K) Memory

| Memory | Size | Contents |
| --- | --- | --- |
| BRAM (828 Kbit) | ~103 KB | Color palette LUT (768 bytes), sprite tile cache, line buffers, SPI receive buffer |
| SIP SDRAM (64 Mbit) | 8 MB | Frame buffer A (300 KB) + Frame buffer B (300 KB) + sprite sheet data + village tilemap |

### Frame Buffer Budget

| Item | Size | Running Total |
| --- | --- | --- |
| Frame Buffer A (640×480×8bit) | 300 KB | 300 KB |
| Frame Buffer B (double buffer) | 300 KB | 600 KB |
| Sprite sheet storage | ~2 MB | 2.6 MB |
| Village tilemap | ~500 KB | 3.1 MB |
| **Free headroom** | **4.9 MB** | **8 MB total** |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## FPGA Modules (All in Verilog)

| # | Module | LUT Estimate | Description |
| --- | --- | --- | --- |
| 1 | `vga_timing.v` | ~200 | H/V counters, HSYNC, VSYNC, video_active |
| 2 | `tmds_encoder.v` | ~600 | 3× TMDS channels (R, G, B) + clock, 8b→10b |
| 3 | `spi_slave.v` | ~300 | Receives binary commands from ESP32-S3 |
| 4 | `cmd_decoder.v` | ~500 | FSM parsing draw commands (sprite, line, clear, text) |
| 5 | `blitter.v` | ~800 | Block transfer engine for sprite rendering |
| 6 | `rasterizer.v` | ~400 | Bresenham's line + rectangle fill |
| 7 | `sdram_ctrl.v` | ~1500 | SDRAM init, refresh, read/write state machine |
| 8 | `palette_lut.v` | ~100 | 256-entry CLUT, 8-bit index → 24-bit RGB |
| 9 | `gpu_top.v` | — | Top-level wiring everything together |
|  | **TOTAL** | **~4,400** | **Uses ~21% of 20,736 LUTs** ✅ plenty of room |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## Communication Protocol (ESP32-S3 → FPGA)

### SPI Command Format

┌──────────┬──────────┬─────────────────────┐
│ CMD (8b) │ LEN (8b) │ PAYLOAD (N bytes)   │
└──────────┴──────────┴─────────────────────┘

| Command | Code | Payload | Description |
| --- | --- | --- | --- |
| NOP | `0x00` | — | No operation |
| CLEAR | `0x01` | 1 byte (color index) | Fill entire screen with palette color |
| DRAW_SPRITE | `0x02` | x(16b) + y(16b) + sprite_id(16b) + flags(8b) | Blit a sprite from SDRAM to framebuffer |
| DRAW_LINE | `0x03` | x0(16b) + y0(16b) + x1(16b) + y1(16b) + color(8b) | Bresenham line |
| DRAW_RECT | `0x04` | x(16b) + y(16b) + w(16b) + h(16b) + color(8b) | Filled rectangle |
| SET_PALETTE | `0x05` | index(8b) + R(8b) + G(8b) + B(8b) | Update one palette entry |
| SWAP_BUFFER | `0x06` | — | Swap front/back framebuffer (vsync) |
| LOAD_SPRITE | `0x07` | addr(24b) + data(Nb) | Upload sprite data to SDRAM |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## Game Data Flow (One Frame)

1. ESP32-S3 Core 1 polls gamepad (USB HID) → player presses D-pad right
2. Game logic moves player sprite X position +2 pixels
3. If player is near NPC and presses A → trigger dialogue
4. Core 0 runs nanoGPT inference → generates NPC response (~1-2 sec)
5. Core 1 sends SPI commands to FPGA:
   a. CLEAR(sky_blue)           — clear back buffer
   b. DRAW_SPRITE(bg_tiles...)  — draw village background
   c. DRAW_SPRITE(npc_id, x, y) — draw NPC sprite
   d. DRAW_SPRITE(player, x, y) — draw player sprite
   e. DRAW_RECT(dialogue_box)   — draw dialogue box
   f. [text rendering commands]  — draw NPC response text
   g. SWAP_BUFFER               — flip to display
6. FPGA reads back buffer from SDRAM, converts through palette LUT
7. TMDS encoder serializes RGB → HDMI output to monitor
8. Repeat at 60 FPS

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## Software Stack

| Layer | Tool | Purpose |
| --- | --- | --- |
| FPGA Synthesis | **Gowin EDA** | Compile Verilog → bitstream for GW2AR-18 |
| FPGA Simulation | **Icarus Verilog + GTKWave** | Pre-synthesis timing verification |
| ESP32-S3 Firmware | **Arduino IDE / PlatformIO** | Game logic + AI inference in C |
| AI Training | **nanoGPT (Python, PyTorch)** | Train 5M param model on dialogue corpus |
| AI Quantization | **ESP-DL / TFLite Micro** | Convert to INT8 for embedded deployment |
| Gamepad Library | **EspUsbHost** | USB HID gamepad reading on ESP32-S3 |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

# Part 2: nanoGPT AI System

# nanoGPT AI System — Complete Plan

"Find the Thief" NPC Dialogue Engine

Trained by us, runs on the ESP32-S3, makes NPCs feel alive.

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## 1. What We're Building

A tiny transformer language model (~5M parameters) that generates contextual NPC dialogue responses based on:

Who the NPC is (personality)

How much they trust the player (trust level 0–100)

What they've seen/experienced (memory)

What the player just said/did (current input)

This replaces hundreds of hand-written dialogue trees with a single model that generates responses.

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## 2. Model Architecture

We're using nanoGPT by Andrej Karpathy — a minimal, clean GPT-2 implementation in PyTorch.

### Configuration for ~5M Parameters

# config/train_thief.py — nanoGPT training config

# Model architecture
n_layer = 4          # 4 transformer blocks (lightweight but capable)
n_head = 6           # 6 attention heads
n_embd = 192         # 192-dimensional embeddings
block_size = 256     # 256 character context window
dropout = 0.1        # 10% dropout for regularization

# Tokenization
# Character-level (NOT BPE) — simpler, smaller vocab, perfect for embedded
# Vocab size ≈ 80-90 characters (letters, digits, punctuation, special tokens)

# Training
batch_size = 32
learning_rate = 3e-4
max_iters = 10000    # ~30 min on a laptop GPU
eval_interval = 500
eval_iters = 200
device = 'cuda'      # or 'cpu' if no GPU (slower but works)

### Parameter Breakdown

| Component | Formula | Parameters |
| --- | --- | --- |
| Token Embedding | vocab_size × n_embd | 90 × 192 = **17,280** |
| Position Embedding | block_size × n_embd | 256 × 192 = **49,152** |
| Per Transformer Block | 12 × n_embd² + 13 × n_embd | 12 × 36,864 + 2,496 = **444,864** |
| × 4 blocks |  | **1,779,456** |
| LayerNorm (final) | 2 × n_embd | **384** |
| Output Head | n_embd × vocab_size | 192 × 90 = **17,280** |
| **TOTAL** |  | **~1.86M** |

### Final Recommended Config

# FINAL — ~4.8M parameters
n_layer = 6
n_head = 8
n_embd = 256
block_size = 256     # 256 chars context (enough for prompt + response)
vocab_size = 90      # character-level

| Metric | Value |
| --- | --- |
| Parameters | ~4.8 million |
| FP32 size | ~19 MB |
| **INT8 quantized size** | **~4.8 MB** ← fits in ESP32-S3 PSRAM |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## 3. Training Data — How I Help You

This is where I come in. You'll ask me to generate the training corpus, and I'll produce hundreds of structured dialogue samples.

### Training Data Format

Each training example follows this exact structure:

<|npc|>SHOPKEEPER<|trust|>HIGH<|memory|>player helped organize shop<|mood|>friendly<|input|>Did you see anything suspicious last night?<|response|>Ah, you've been so kind to me, friend. I'll be honest — I heard footsteps near the vault around midnight. Couldn't see who it was, but they were heading toward the old well.<|end|>

### The Prompt Template

<|npc|>{NPC_NAME}<|trust|>{LOW/MEDIUM/HIGH}<|memory|>{what this NPC remembers}<|mood|>{current emotional state}<|input|>{what the player said or did}<|response|>

The model learns to complete everything after <|response|> and stop at <|end|>.

### NPC Profiles (What I'll Bake Into the Training Data)

#### Shopkeeper (Ravi)

Personality: Friendly but calculative, knows everyone's business, gossips

Secret: He's the actual thief (or red herring — your choice)

Speech style: Warm, uses phrases like "my friend," deflects accusations smoothly

Low trust: Gives generic helpful advice, hides information

High trust: Reveals secrets about other NPCs, may slip up about the theft

#### Guard (Karan)

Personality: Stern, proud, takes his job seriously, gets angry if accused

Secret: Was asleep on duty the night of the theft (embarrassed about it)

Speech style: Formal, short sentences, military-like

Low trust: Dismissive, "That's none of your concern, citizen"

High trust: Admits he fell asleep, shares patrol route details

#### Old Man (Thakur Ji)

Personality: Wise, cryptic, speaks in riddles/metaphors, knows village history

Secret: Saw something from his window but is afraid to speak

Speech style: Slow, philosophical, "In my day..." type stories

Low trust: Tells irrelevant old stories, avoids the topic

High trust: Reveals what he saw — a shadow carrying something heavy

#### Kid (Chintu)

Personality: Curious, energetic, sneaks around at night, sees everything

Secret: Saw the shopkeeper carrying a bag near the vault

Speech style: Excited, uses "bhaiya" / informal language, easily distracted

Low trust: Talks about games and random stuff, doesn't help

High trust: Blurts out what he saw — crucial clue

#### Farmer (Baldev)

Personality: Hardworking, quiet, suspicious of outsiders, honest

Secret: His tools were stolen too (possibly used in the heist)

Speech style: Simple, direct, few words

Low trust: "I have work to do. Leave me alone."

High trust: Mentions his missing tools, connects dots

### Training Data Volume Target

| Category | Count |
| --- | --- |
| NPCs | 5 |
| Trust levels | 3 (LOW, MEDIUM, HIGH) |
| Scenario types | ~10 (greeting, accusation, asking for help, giving clue, refusing, etc.) |
| Variations per scenario | 3–5 |
| **Total training samples** | **~500–750 dialogues** |
| Characters per sample | ~200–400 |
| **Total training text** | **~150,000–250,000 characters** |

### How We Generate This Together

Step 1: I generate the base corpus. You tell me:

"Generate 50 training dialogues for the Shopkeeper at all trust levels"

And I produce 50 structured samples like:

<|npc|>SHOPKEEPER<|trust|>LOW<|memory|>none<|mood|>neutral<|input|>Hello, I'm investigating the theft.<|response|>Welcome, welcome! Terrible business, that theft. I hope you find the culprit. Can I interest you in anything from my shop?<|end|>

<|npc|>SHOPKEEPER<|trust|>LOW<|memory|>none<|mood|>nervous<|input|>Where were you last night?<|response|>Me? I was right here, counting inventory. Ask anyone! I'm always here. Now, is there something you'd like to buy?<|end|>

<|npc|>SHOPKEEPER<|trust|>HIGH<|memory|>player helped organize shop,player bought items<|mood|>grateful<|input|>I need to know the truth. Did you take the gold?<|response|>You wound me, friend! After all we've shared? But... I'll tell you something. I saw the guard's post was empty that night. Completely unattended. Make of that what you will.<|end|>

Step 2: You review/edit them for tone and accuracy.

Step 3: We repeat for all 5 NPCs.

Step 4: I also generate "edge case" samples — wrong accusations, repeated questions, contradictions.

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## 4. Training Pipeline (On Your PC)

### Prerequisites

# Install Python (if not already installed)
# Download from python.org — make sure to check "Add to PATH"

# Clone nanoGPT
cd "D:\CP GPU"
git clone https://github.com/karpathy/nanoGPT.git
cd nanoGPT

# Install dependencies
pip install torch numpy transformers datasets tiktoken tqdm

### Step-by-Step Training

D:\CP GPU\nanoGPT\
├── data/
│   └── find_the_thief/
│       ├── input.txt          ← All 500+ dialogues concatenated
│       └── prepare.py         ← Character-level tokenizer script
├── config/
│   └── train_thief.py         ← Our custom training config
├── train.py                   ← nanoGPT trainer (already in repo)
└── sample.py                  ← Text generation script (already in repo)

Step 1: Prepare the data

# After I generate all dialogues and you save them to input.txt:
cd "D:\CP GPU\nanoGPT"
python data/find_the_thief/prepare.py
# This creates train.bin and val.bin

Step 2: Train

python train.py config/train_thief.py
# ~30 minutes on a laptop GPU (GTX 1650 or better)
# ~2-4 hours on CPU (still doable)
# Outputs: out-thief/ckpt.pt (the trained model)

Step 3: Test generation

python sample.py --out_dir=out-thief --start="<|npc|>GUARD<|trust|>LOW<|memory|>none<|mood|>angry<|input|>You fell asleep on duty!<|response|>"
# Should generate a Guard-like angry denial response

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## 5. Deployment Pipeline (PC → ESP32-S3)

graph LR
    A["PyTorch Model<br/>ckpt.pt (~19 MB)"] -->|"torch.onnx.export()"| B["ONNX Model<br/>model.onnx"]
    B -->|"ESP-PPQ Quantizer"| C["INT8 Model<br/>model_int8.bin (~4.8 MB)"]
    C -->|"Copy to SD card"| D["microSD Card"]
    D -->|"Loaded at boot"| E["ESP32-S3 PSRAM<br/>(Runtime Inference)"]

### Step-by-Step Conversion

# export_model.py — Run on your PC after training

import torch
from model import GPT, GPTConfig

# 1. Load trained model
config = GPTConfig(
    n_layer=6, n_head=8, n_embd=256,
    block_size=256, vocab_size=90
)
model = GPT(config)
checkpoint = torch.load('out-thief/ckpt.pt', map_location='cpu')
model.load_state_dict(checkpoint['model'])
model.eval()

# 2. Export to ONNX
dummy_input = torch.randint(0, 90, (1, 256))  # batch=1, seq=256
torch.onnx.export(
    model, dummy_input, "model.onnx",
    input_names=['input_ids'],
    output_names=['logits'],
    opset_version=13
)
print("Exported to ONNX!")

# 3. Quantize with ESP-PPQ (Espressif's quantization tool)
# pip install esp-ppq
# python -m esp_ppq quantize --input model.onnx --output model_int8.bin --target esp32s3

### SD Card Structure

microSD Card (FAT32)
├── model/
│   ├── model_int8.bin        ← 4.8 MB quantized weights
│   ├── vocab.bin             ← Character-to-integer mapping
│   └── config.bin            ← Model config (layers, dims, etc.)
├── sprites/
│   ├── shopkeeper.bin        ← Sprite data for FPGA
│   ├── guard.bin
│   └── ...
└── maps/
    └── village.bin           ← Tilemap data

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## 6. Runtime on ESP32-S3

### Dual-Core Architecture

┌─────────────────────────────────────────────────────────┐
│                    ESP32-S3 Runtime                      │
│                                                         │
│  ┌───────────────────────┐  ┌────────────────────────┐  │
│  │ CORE 1 (Game Loop)    │  │ CORE 0 (AI Inference)  │  │
│  │ Priority: REALTIME    │  │ Priority: NORMAL       │  │
│  │                       │  │                        │  │
│  │ • Poll gamepad (USB)  │  │ • Idle until triggered  │  │
│  │ • Update game state   │  │ • Build prompt string  │  │
│  │ • Collision detection │  │ • Tokenize (char→int)  │  │
│  │ • Send draw cmds SPI  │  │ • Load weights from    │  │
│  │ • Manage NPC states   │  │   PSRAM (pre-loaded    │  │
│  │ • Trigger AI when     │  │   from SD at boot)     │  │
│  │   player talks to NPC │  │ • Run transformer      │  │
│  │                       │  │   layer by layer       │  │
│  │ ──── NEVER BLOCKS ─── │  │ • Generate ~30-60      │  │
│  │   (always 60 FPS)     │  │   characters           │  │
│  │                       │  │ • Return response to   │  │
│  │                       │  │   Core 1 via queue     │  │
│  └───────────────────────┘  └────────────────────────┘  │
│                                                         │
│  ┌─────────────────────────────────────────────────────┐ │
│  │                    PSRAM (8 MB)                      │ │
│  │  ┌──────────────┐  ┌────────────────┐  ┌─────────┐ │ │
│  │  │ Model Weights│  │ Inference      │  │ Game    │ │ │
│  │  │ ~4.8 MB      │  │ Workspace ~1MB │  │ State   │ │ │
│  │  └──────────────┘  └────────────────┘  └─────────┘ │ │
│  └─────────────────────────────────────────────────────┘ │
└─────────────────────────────────────────────────────────┘

### Prompt Construction at Runtime

When the player talks to an NPC, Core 1 sends this to Core 0:

// game_ai.h — Prompt builder

typedef struct {
    char npc_name[16];      // "SHOPKEEPER"
    uint8_t trust_level;    // 0-100 → mapped to LOW/MEDIUM/HIGH
    char memory[128];       // "player helped organize shop"
    char mood[16];          // "friendly"
    char player_input[128]; // "Did you see anything?"
} AIRequest;

// Core 1 builds and sends:
AIRequest req = {
    .npc_name = "KID",
    .trust_level = 75,       // HIGH
    .memory = "player gave him candy,player played catch",
    .mood = "excited",
    .player_input = "Did you see anything the night the gold disappeared?"
};
xQueueSend(ai_queue, &req, portMAX_DELAY);

// Core 0 constructs the prompt string:
// "<|npc|>KID<|trust|>HIGH<|memory|>player gave him candy,player played catch<|mood|>excited<|input|>Did you see anything the night the gold disappeared?<|response|>"
// Then runs inference character-by-character until <|end|> token

### Expected Performance

| Metric | Value |
| --- | --- |
| Model load time (SD → PSRAM) | ~2 seconds at boot |
| Prompt tokenization | <1 ms |
| Inference per token | ~30–50 ms |
| Typical response length | 30–60 characters |
| **Total response time** | **~1–3 seconds** |
| Player perception | Shows "..." animation while thinking → feels natural |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## 7. Fallback System (Safety Net)

If the AI model is too slow or produces garbage, we have a backup:

// fallback_dialogue.h — Pre-written responses indexed by NPC + trust + scenario

const char* fallback_responses[5][3][10] = {
    // [NPC_SHOPKEEPER][TRUST_LOW][SCENARIO_GREETING]
    { { "Welcome to my shop! Looking to buy something?",
        "Ah, a visitor! Browse around, take your time.",
        ... } },
    // [NPC_SHOPKEEPER][TRUST_HIGH][SCENARIO_ACCUSATION]
    { { "After all I've done for you? I'm hurt, truly hurt.",
        ... } },
    ...
};

// Use fallback if:
// 1. AI inference exceeds 5 second timeout
// 2. Generated text is too short (<10 chars) or too long (>200 chars)
// 3. Generated text doesn't end with <|end|> token

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## 8. Timeline for the AI Component

| Phase | Task | Time | Who Does It |
| --- | --- | --- | --- |
| **Phase 1** | I generate 500+ dialogue samples | 1 session | You ask me, I produce |
| **Phase 2** | You review/edit the dialogues | 1–2 days | You |
| **Phase 3** | Install nanoGPT + train model | 1 day | You (I guide step-by-step) |
| **Phase 4** | Test generation on PC | 1 day | You (I help debug) |
| **Phase 5** | Quantize + deploy to ESP32-S3 | 2–3 days | We do together |
| **Phase 6** | Integrate with game loop | 2–3 days | We do together |
| **Phase 7** | Test + tune | Ongoing | You play, we iterate |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

# Part 3: Game Development Plan

# Game Building Plan — "Find the Thief"

## 1. Game Overview

A 2D top-down mystery game set in a pixel-art village. The player walks around, talks to 5 NPCs, gathers clues, and accuses the thief. NPC responses are generated by a nanoGPT AI model running on the ESP32-S3.

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## 2. Art Pipeline — Creating Characters & World

### Tools Required

| Tool | Purpose | Cost |
| --- | --- | --- |
| **Aseprite** | Pixel art sprites + animations | $20 (Steam) or free via source compile |
| **Tiled Map Editor** | Village tilemap design | Free (mapeditor.org) |
| **Python (Pillow)** | Convert PNG sprites → raw binary for FPGA | Free |

### Sprite Specifications

| Asset | Size | Frames | Color Depth |
| --- | --- | --- | --- |
| Player character | 16×32 px | 4 per direction (walk) + 1 idle = 17 frames | 8-bit indexed |
| Each NPC (×5) | 16×32 px | 2 idle animation frames each | 8-bit indexed |
| Village tiles | 16×16 px | ~40 unique tiles (grass, path, wall, water, roof, door, fence, tree) | 8-bit indexed |
| Dialogue box | 280×80 px | 1 static frame | 8-bit indexed |
| Font glyphs | 8×8 px | 96 ASCII characters (A-Z, a-z, 0-9, punctuation) | 1-bit (monochrome) |
| UI elements | Various | Cursor, trust meter, accuse button | 8-bit indexed |

### Character Design Guide

Each NPC needs a visually distinct silhouette even at 16×32:

| NPC | Visual Identity | Key Color |
| --- | --- | --- |
| Shopkeeper (Ravi) | Apron, round body, mustache | Warm brown/orange |
| Guard (Karan) | Helmet, spear, upright posture | Steel blue/grey |
| Old Man (Thakur Ji) | Walking stick, hunched, long beard | Muted green/white |
| Kid (Chintu) | Small height (16×24), cap, energetic pose | Bright yellow/red |
| Farmer (Baldev) | Straw hat, broad shoulders, hoe on back | Earth brown/green |

### Art Creation Workflow

Step 1: Create a 256-color palette in Aseprite (shared across all sprites)
        → Export as palette.pal

Step 2: Draw each character on separate Aseprite files
        → Use Onion Skinning for animation frames
        → Export each as a sprite sheet PNG (horizontal strip)

Step 3: Draw all 16×16 tiles in one Aseprite tileset file
        → Export as tileset.png

Step 4: Open Tiled Map Editor → import tileset.png
        → Paint the village map (40×30 tiles = 640×480 pixels)
        → Mark collision zones and NPC spawn points
        → Export as village.json

Step 5: Run conversion script (Python) to convert everything to binary:
        → sprite PNGs → .bin files (raw palette indices)
        → village.json → village.bin (tile index array)
        → Copy all .bin files to microSD card

### Sprite-to-Binary Conversion Script

# tools/convert_sprites.py
from PIL import Image
import struct

def png_to_bin(input_png, output_bin, palette):
    """Convert indexed-color PNG to raw binary for FPGA."""
    img = Image.open(input_png).convert('P')  # Palette mode
    pixels = list(img.getdata())
    with open(output_bin, 'wb') as f:
        f.write(struct.pack('<HH', img.width, img.height))  # Header
        f.write(bytes(pixels))  # Raw palette indices
    print(f"Converted {input_png} → {output_bin} ({len(pixels)} bytes)")

def convert_tilemap(json_path, output_bin):
    """Convert Tiled JSON export to binary tilemap."""
    import json
    with open(json_path) as f:
        data = json.load(f)
    layer = data['layers'][0]  # Ground layer
    tiles = layer['data']
    w, h = layer['width'], layer['height']
    with open(output_bin, 'wb') as f:
        f.write(struct.pack('<HH', w, h))
        for tile_id in tiles:
            f.write(struct.pack('<B', tile_id & 0xFF))
    print(f"Converted tilemap: {w}×{h} tiles → {output_bin}")

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## 3. Game State Machine

The game runs on ESP32-S3 Core 1 as a finite state machine:

┌──────────┐
                    │  BOOT    │
                    │ Load AI  │
                    │ Load Art │
                    └────┬─────┘
                         │
                    ┌────▼─────┐
              ┌────►│EXPLORING │◄────────────────┐
              │     │ Walk map │                  │
              │     │ See NPCs │                  │
              │     └────┬─────┘                  │
              │          │ Press A near NPC        │
              │     ┌────▼──────┐                 │
              │     │DIALOGUE   │                 │
              │     │ AI runs   │                 │
              │     │ Show text │                 │
              │     └────┬──────┘                 │
              │          │ Press B to close        │
              │          ├─────────────────────────┘
              │          │ Press X to accuse
              │     ┌────▼──────┐
              │     │ACCUSATION │
              │     │ Confirm?  │
              │     └────┬──────┘
              │          │
              │     ┌────▼──────┐
              │     │GAME OVER  │
              │     │ Win/Lose  │
              └─────┤ Retry?    │
                    └───────────┘

### State Descriptions

| State | What Happens | Inputs | FPGA Rendering |
| --- | --- | --- | --- |
| **BOOT** | Load AI model from SD to PSRAM, load sprite data to FPGA SDRAM | None (automatic) | Splash screen |
| **EXPLORING** | Player walks around village with D-pad, camera follows | D-pad: move, A: interact | Village tilemap + all NPC sprites + player sprite |
| **DIALOGUE** | AI generates NPC response, text appears character-by-character | A: next line, B: close | Village + dialogue box overlay + NPC portrait |
| **ACCUSATION** | Player selects an NPC to accuse as the thief | D-pad: select, A: confirm, B: cancel | NPC lineup + selection cursor |
| **GAME_OVER** | Win (correct accusation) or Lose (wrong accusation) | A: retry | Result screen with story summary |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## 4. ESP32-S3 Game Engine Code Structure

D:\CP GPU\firmware\
├── main/
│   ├── main.c              ← Entry point, FreeRTOS task creation
│   ├── game_loop.c         ← Core 1: state machine, 60 FPS tick
│   ├── game_states.h       ← State enum + transition logic
│   ├── player.c            ← Player movement, collision, position
│   ├── npc.c               ← NPC data: position, trust, memory, mood
│   ├── npc.h               ← NPC struct definitions
│   ├── dialogue.c          ← Dialogue box rendering commands
│   ├── input.c             ← USB HID gamepad polling (EspUsbHost)
│   ├── gpu_commands.c      ← SPI command builder (DRAW_SPRITE, etc.)
│   ├── gpu_commands.h      ← Command codes and packet structs
│   ├── ai_inference.c      ← Core 0: nanoGPT INT8 inference engine
│   ├── ai_inference.h      ← AIRequest struct, queue interface
│   ├── asset_loader.c      ← SD card → PSRAM/FPGA asset loading
│   └── map.c               ← Tilemap loader, collision map, camera
├── components/
│   ├── esp_usb_host/       ← EspUsbHost library
│   └── esp_dl/             ← ESP-DL inference framework
└── sdkconfig               ← ESP-IDF configuration

### Core Game Loop (Pseudocode)

// game_loop.c — Runs on Core 1 at 60 FPS

void game_loop_task(void *param) {
    GameState state = STATE_BOOT;
    Player player = {.x = 320, .y = 240, .sprite_id = 0};
    NPC npcs[5];
    init_npcs(npcs);

    while (1) {
        uint32_t frame_start = millis();
        GamepadInput input = poll_gamepad();

        switch (state) {
            case STATE_BOOT:
                gpu_cmd_clear(COLOR_BLACK);
                gpu_cmd_draw_sprite(SPLASH_SCREEN, 0, 0);
                gpu_cmd_swap_buffer();
                load_ai_model();        // SD → PSRAM
                load_sprites_to_fpga(); // SD → SPI → FPGA SDRAM
                state = STATE_EXPLORING;
                break;

            case STATE_EXPLORING:
                update_player(&player, input);
                check_npc_proximity(&player, npcs, &state);

                // Render frame
                gpu_cmd_clear(COLOR_SKY);
                draw_tilemap(player.camera_x, player.camera_y);
                for (int i = 0; i < 5; i++)
                    gpu_cmd_draw_sprite(npcs[i].sprite_id, npcs[i].x, npcs[i].y);
                gpu_cmd_draw_sprite(player.sprite_id, player.screen_x, player.screen_y);
                gpu_cmd_swap_buffer();
                break;

            case STATE_DIALOGUE:
                if (ai_response_ready()) {
                    char *response = get_ai_response();
                    draw_dialogue_box(response, current_npc);
                }
                if (input.b_pressed) state = STATE_EXPLORING;
                if (input.x_pressed) state = STATE_ACCUSATION;
                gpu_cmd_swap_buffer();
                break;

            case STATE_ACCUSATION:
                draw_accusation_screen(npcs);
                if (input.a_pressed) {
                    if (selected_npc == THE_THIEF)
                        state = STATE_WIN;
                    else
                        state = STATE_LOSE;
                }
                break;
        }

        // Frame rate limiter: 16.67ms per frame = 60 FPS
        uint32_t elapsed = millis() - frame_start;
        if (elapsed < 16) vTaskDelay(pdMS_TO_TICKS(16 - elapsed));
    }
}

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## 5. NPC Interaction System

### Trust Mechanics

| Action | Trust Change |
| --- | --- |
| Help an NPC (do their side task) | +20 |
| Give an item they want | +15 |
| Talk politely | +5 |
| Wrongly accuse them | -30 |
| Ask rude questions | -10 |
| Ignore them for a long time | -5 |

### Trust Level Thresholds

| Range | Label | NPC Behavior |
| --- | --- | --- |
| 0–30 | LOW | Dismissive, unhelpful, generic responses |
| 31–65 | MEDIUM | Somewhat helpful, gives vague hints |
| 66–100 | HIGH | Shares secrets, reveals clues, trusts player |

### Side Tasks (Trust Builders)

| NPC | Side Task | Reward |
| --- | --- | --- |
| Shopkeeper | Help organize crates (mini-game or auto) | +20 trust with Shopkeeper |
| Guard | Find his missing badge | +20 trust with Guard |
| Old Man | Bring him his medicine from the shop | +20 trust with Old Man |
| Kid | Play catch / find his lost toy | +20 trust with Kid |
| Farmer | Water his crops (carry bucket from well) | +20 trust with Farmer |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## 6. Village Map Layout

┌──────────────────────────────────────────────────┐
│  TREES  TREES  TREES  TREES  TREES  TREES  TREES│
│  TREES ┌──────┐    PATH    ┌──────┐  TREES      │
│        │SHOP  │────────────│GUARD │              │
│        │      │    WELL    │POST  │              │
│        └──────┘     ○      └──────┘              │
│                     │                             │
│    PATH─────────PATH+PATH──────────PATH           │
│                     │                             │
│  ┌──────┐          │           ┌──────┐          │
│  │OLD   │          │           │FARM  │          │
│  │MAN   │     ┌────┴────┐     │HOUSE │          │
│  │HOUSE │     │ VILLAGE │     │      │          │
│  └──────┘     │ CENTER  │     └──────┘          │
│               │(PLAYER  │                        │
│               │ START)  │     CROPS CROPS        │
│               └─────────┘     CROPS CROPS        │
│                                                   │
│        ┌───────┐                                 │
│        │KID's  │    RIVER ~~~~~~~~               │
│        │HIDING │    RIVER ~~~~~~~~               │
│        │SPOT   │                                 │
│        └───────┘                                 │
│  TREES  TREES  TREES  TREES  TREES  TREES  TREES│
└──────────────────────────────────────────────────┘

Map: 40×30 tiles (16px each) = 640×480 pixels (exactly fills the screen — no scrolling needed for MVP).

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## 7. Development Phases — Start to Finish

### Phase 1: Foundation (Week 1–2)

| Task | Details | Output |
| --- | --- | --- |
| Install all tools | Aseprite, Tiled, Gowin EDA, Icarus Verilog, Arduino IDE | Working dev environment |
| Create color palette | 256-color palette shared by all art assets | palette.pal |
| Draw player sprite | 16×32, idle + 4-direction walk animation | player.aseprite → player.png |
| Draw 5 NPC sprites | 16×32 each, idle animation (2 frames) | npc_*.png |
| Draw village tileset | 16×16 tiles (~40 tiles) | tileset.png |
| Design village map | In Tiled, place tiles, mark collision, NPC spawns | village.json |

### Phase 2: FPGA GPU Core (Week 2–4)

| Task | Details | Output |
| --- | --- | --- |
| Simulate VGA timing | Verify HSYNC/VSYNC/video_active in GTKWave | Simulation screenshots |
| Implement SDRAM controller | Use nand2mario's open-source controller as reference | sdram_ctrl.v |
| Implement TMDS encoder | 3-channel 8b→10b encoding for HDMI | tmds_encoder.v |
| Implement palette LUT | 256-entry BRAM color lookup | palette_lut.v |
| Build SPI slave | Receive commands from ESP32-S3 | spi_slave.v |
| Build command decoder | Parse CLEAR, DRAW_SPRITE, SWAP_BUFFER, etc. | cmd_decoder.v |
| Build blitter | Transfer sprite data from SDRAM to framebuffer | blitter.v |
| Synthesize + flash | Gowin EDA → bitstream → flash to Tang Nano 20K | gpu.fs |
| Test: solid color on HDMI | Send CLEAR command, see solid color on monitor | First visual output |

### Phase 3: ESP32-S3 Game Engine (Week 3–5)

| Task | Details | Output |
| --- | --- | --- |
| USB gamepad input | EspUsbHost library, read D-pad + buttons | input.c |
| SPI master driver | Send draw commands to FPGA at high speed | gpu_commands.c |
| SD card asset loader | Read sprite .bin and map .bin from SD | asset_loader.c |
| Player movement | D-pad → position update with collision | player.c |
| Tilemap renderer | Send tile draw commands for visible area | map.c |
| NPC interaction | Proximity detection, trigger dialogue | npc.c |
| State machine | BOOT → EXPLORING → DIALOGUE → ACCUSATION → GAME_OVER | game_loop.c |

### Phase 4: AI Integration (Week 5–7)

| Task | Details | Output |
| --- | --- | --- |
| Generate training data | I produce 500+ dialogue samples, you review | input.txt |
| Train nanoGPT | Clone repo, prepare data, train ~30 min on GPU | ckpt.pt |
| Quantize model | PyTorch → ONNX → INT8 via ESP-PPQ | model_int8.bin |
| ESP32-S3 inference | Load model from SD, run on Core 0 | ai_inference.c |
| Dialogue box rendering | Character-by-character text display with "..." animation | dialogue.c |
| Trust system | Track per-NPC trust, map to LOW/MEDIUM/HIGH | npc.c |

### Phase 5: Polish & Integration (Week 7–8)

| Task | Details | Output |
| --- | --- | --- |
| Side tasks | Implement mini-tasks for each NPC | side_tasks.c |
| Accusation system | NPC lineup, selection, win/lose logic | accusation.c |
| Game over screen | Win message + story recap / Lose message + retry | game_over.c |
| Sound (optional) | Tang Nano 20K has MAX98357A audio amp | audio.v |
| Bug fixes + tuning | Playtesting, timing fixes, AI response quality | Final build |
| Presentation prep | GTKWave screenshots, live demo, documentation | Presentation materials |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## 8. Asset File Sizes (Estimated)

| Asset | Calculation | Size |
| --- | --- | --- |
| Player sprite (17 frames × 16×32) | 17 × 512 bytes | ~8.5 KB |
| 5 NPC sprites (2 frames × 16×32 each) | 5 × 2 × 512 | ~5 KB |
| Village tileset (40 tiles × 16×16) | 40 × 256 | ~10 KB |
| Village tilemap (40×30 indices) | 1,200 bytes | ~1.2 KB |
| Font glyphs (96 chars × 8×8 / 8) | 96 × 8 | ~768 bytes |
| Dialogue box frame | 280×80 | ~22 KB |
| Color palette (256 × 3 bytes RGB) | 768 bytes | ~768 bytes |
| **Total art assets** |  | **~48 KB** |
| AI model (INT8) |  | ~4.8 MB |
| **Grand total on SD card** |  | **~5 MB** |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

## 9. Testing Checklist

| # | Test | Pass Criteria |
| --- | --- | --- |
| 1 | FPGA shows solid color via HDMI | Monitor displays uniform color |
| 2 | Player sprite renders on screen | Character visible at center |
| 3 | D-pad moves player | Smooth 4-direction movement |
| 4 | Tilemap renders correctly | Village appears as designed in Tiled |
| 5 | NPC sprites visible at correct positions | All 5 NPCs in their map locations |
| 6 | Pressing A near NPC triggers dialogue | Dialogue box appears |
| 7 | AI generates contextual response | Response matches NPC personality + trust |
| 8 | Trust changes with player actions | Helping NPC increases trust, accusations decrease |
| 9 | Accusation flow works | Correct accusation = win, wrong = lose |
| 10 | Stable 60 FPS during exploration | No visible stuttering or tearing |